Yes. Your teacher has given 4 separate requirements (i, ii, iii, iv). In the assignment report, you should answer them one by one.

Assignment 01: IP Addressing Using Subnetting Technique

(i) Solve the IP Addressing Using Subnetting Technique

To efficiently allocate IP addresses according to the number of hosts in each LAN and WAN, the Variable Length Subnet Masking (VLSM) technique is used. VLSM allows different subnet sizes based on host requirements, reducing IP address wastage.

The host requirements are:

| Network | Hosts |
| --- | --- |
| LAN-4 | 5000 |
| LAN-2 | 4096 |
| LAN-5 | 1096 |
| LAN-3 | 1024 |
| LAN-1 | 127 |
| WAN-1 to WAN-5 | 2 hosts each |

The subnets are allocated from largest to smallest to achieve efficient address utilization.

(ii) Choose an Appropriate Classful IP Address

Since the network requires more than 10,000 hosts in total, a Class C network is insufficient.

A Class B private network is selected:

Network Address: 172.16.0.0

Default Mask: 255.255.0.0 (/16)

Reasons for choosing Class B:

- Supports a large number of hosts.

- Provides enough address space for future expansion.

- Suitable for VLSM implementation.

- Prevents frequent subnet exhaustion.

(iii) Efficiently Allocate the Chosen IP Address to Prevent Wastage

VLSM is used to allocate subnet sizes according to actual host requirements.

| Network | Hosts Required | Allocated Prefix |
| --- | --- | --- |
| LAN-4 | 5000 | /19 |
| LAN-2 | 4096 | /19 |
| LAN-5 | 1096 | /21 |
| LAN-3 | 1024 | /21 |
| LAN-1 | 127 | /24 |
| WAN Links | 2 | /30 |

This method minimizes unused addresses because each subnet receives only the address space it needs.

(iv) Derive Network Address, Broadcast Address and Subnet Mask

| Network | Network Address | Prefix | Subnet Mask | Broadcast Address |
| --- | --- | --- | --- | --- |
| LAN-4 | 172.16.0.0 | /19 | 255.255.224.0 | 172.16.31.255 |
| LAN-2 | 172.16.32.0 | /19 | 255.255.224.0 | 172.16.63.255 |
| LAN-5 | 172.16.64.0 | /21 | 255.255.248.0 | 172.16.71.255 |
| LAN-3 | 172.16.72.0 | /21 | 255.255.248.0 | 172.16.79.255 |
| LAN-1 | 172.16.80.0 | /24 | 255.255.255.0 | 172.16.80.255 |
| WAN-1 | 172.16.81.0 | /30 | 255.255.255.252 | 172.16.81.3 |
| WAN-2 | 172.16.81.4 | /30 | 255.255.255.252 | 172.16.81.7 |
| WAN-3 | 172.16.81.8 | /30 | 255.255.255.252 | 172.16.81.11 |
| WAN-4 | 172.16.81.12 | /30 | 255.255.255.252 | 172.16.81.15 |
| WAN-5 | 172.16.81.16 | /30 | 255.255.255.252 | 172.16.81.19 |

Conclusion

A Class B network (172.16.0.0/16) was selected and subnetted using the VLSM technique. The allocated subnet sizes satisfy all LAN and WAN requirements while minimizing address wastage. The resulting design is scalable, efficient, and suitable for enterprise-level networking.