Here is how your PDFs perfectly mapped out to complete your syllabus:

- Group A (Basic Concepts, ALOHA, Piggybacking, CSMA): Fully covered using Lecture-1, ch2_v1, Lecture1_Ch11 (for Piggybacking), and Lecture-11 (for ALOHA and CSMA persistence).

- Group B (Stop-and-Wait, Stop-and-Wait ARQ, Go-Back-N, Selective Repeat): Fully covered using Lecture1_Ch11.

- Group C (CSMA/CD, CSMA/CA, Token Passing, Polling, Reservation): Fully covered using Lecture-11.

Group A: Basic Concepts & Protocols

1. Basic Concepts & Topologies

- Data Communications: The exchange of data between two devices via some form of transmission medium.

- System Components: A complete data communication system contains five essential elements: Message, Sender, Receiver, Medium, and Protocol.

- System Effectiveness: Evaluated by four key characteristics: Delivery (reaching the correct destination), Accuracy (correct data), Timeliness (delivered on time), and Jitter (packets arrival time variance).

- Data Flow Modes: Can be Simplex (one-way transmission), Half-Duplex (two-way but not simultaneous), or Full-Duplex (simultaneous two-way transmission).

- Network Criteria: Networks must satisfy three primary parameters: Performance (measured by throughput and delay), Reliability (measured by frequency of failure), and Security (unauthorized access protection).

- Connections & Topologies:

  - Point-to-Point: A dedicated link exists between two and only two devices.

  - Multipoint: Three or more devices share a single common link.

  - Mesh Topology: Every device has a dedicated point-to-point link to every other device, requiring $\frac{n(n-1)}{2}$ physical links for $n$ devices.

  - Star Topology: Each device connects directly to a central controller or hub.

  - Bus Topology: A multipoint layout consisting of a long central cable (backbone) with drop lines and taps.

  - Ring Topology: Each device connects exclusively to its two immediate neighbors in a continuous loop.

2. Network Models & Addressing

- OSI vs. TCP/IP Models: The OSI model features a 7-layer layout, while the TCP/IP protocol suite maps directly to 5 operational layers: Physical, Data Link, Network, Transport, and Application.

- Layer Responsibilities: The Data Link layer splits its focus into Data Link Control (node-to-node communication) and Media Access Control (coordinating access to shared media).

- Addressing Types:

  - Physical Address: A 48-bit (6-byte) local MAC address handled at the Data Link layer for hop-to-hop frame transfers.

  - Logical Address: A 32-bit (IPv4) or 128-bit (IPv6) address handled at the Network layer for global source-to-destination delivery.

  - Port Address: A 16-bit process address handled at the Transport layer to map data to a specific software application.

- IPv4 Subnetting & NAT: Subnetting divides a single large network into smaller ones by extending the network mask prefix. Network Address Translation (NAT) maps internal private IP ranges to a single global public IP address to save address space.

3. Piggybacking

- Definition: An optimization technique used in bidirectional data link communication protocols.

- Mechanism: Instead of creating separate, independent control frames to acknowledge data reception, an acknowledgment (ACK) is attached directly into an outgoing data frame moving in that same direction.

4. ALOHA Protocols

- Pure ALOHA: An early random-access protocol where stations transmit frames immediately whenever they have data ready to send.

  - Collision Rule: If an acknowledgement (ACK) is not received within a timeout period equal to twice the maximum propagation delay ($2 \times T_p$), the station assumes a collision occurred.

  - Back-off Strategy: To recover from a collision, the station waits a random back-off time $T_B = R \times T_p$ or $R \times T_{fr}$ (where $R$ is a random number chosen between $0$ and $2^K-1$, and $K$ is the number of failed attempts) before trying again.

  - Throughput & Vulnerable Time: Pure ALOHA throughput is modeled as $S = G \times e^{-2G}$, reaching a maximum throughput of $18.4\%$ when $G = 1/2$. Its total vulnerable time is equal to $2 \times T_{fr}$.

- Slotted ALOHA: Improves efficiency by dividing continuous channel time into specific slots of $T_{fr}$ seconds, forcing all stations to only transmit exactly at the beginning of a time slot.

  - Throughput & Vulnerable Time: Slotted ALOHA throughput is modeled as $S = G \times e^{-G}$, reaching a maximum efficiency of $36.8\%$ when $G = 1$. Its vulnerable time is successfully reduced down to $1 \times T_{fr}$.

5. CSMA Technique (Persistence Strategies)

- Core Principle: Carrier Sense Multiple Access operates on the rule of "sense before transmit" or "listen before talk" to reduce collisions.

- The Collision Constraint: While CSMA reduces conflicts, collisions still happen due to propagation delay; a station might sense a medium as idle because the first bit of an active transmission hasn't traveled down the wire to it yet. The vulnerable time is equal to the propagation time $T_p$.

- Persistence Methods: Strategies defining what a station does if it senses that the link is busy:

  - 1-Persistent Strategy: If the line is busy, the station continuously monitors it; the moment it finds the line idle, it transmits its frame immediately with a probability of 1.

  - Non-Persistent Strategy: If the line is busy, the station immediately waits a random amount of time and then senses the line all over again (reducing the chance of synchronized collisions).

  - p-Persistent Strategy: Used when time is slotted. If the line is found idle, the station transmits its frame with a probability of $p$. With a probability of $q = 1-p$, it waits for the start of the next slot to re-sense the line.

Group B: Data Link Control & ARQ Protocols

1. Protocols for Noiseless Channels

- Simplest Protocol: Assumes a perfect link where no frames are lost or corrupted. It has no flow or error control; the receiver is assumed to process incoming data instantly without its buffers filling up.

- Stop-and-Wait Protocol: Adds fundamental flow control. The sender transmits a single frame and must completely halt until it receives an ACK frame from the receiver before sending the next packet.

2. Protocols for Noisy Channels (ARQ)

- Stop-and-Wait ARQ: Adds error control to flow control by adding alternating sequence numbers (0 and 1) to frames, using a countdown timer, and keeping a copy of the sent frame. If an ACK doesn't return before a timeout, the frame is resent.

- Go-Back-N ARQ: Allows sending multiple frames before receiving an ACK to keep the link busy.

  - Windows: The Send Window size is bounded to a maximum of $2^m - 1$ (using modulo $2^m$ sequence numbers). The Receive Window is strictly fixed at a size of $1$.

  - Behavior: The receiver only accepts the exact frame it expects next ($R_n$). Any out-of-order frame is immediately dropped. If a timeout happens, the sender must roll back and retransmit the lost frame plus all subsequent unacknowledged frames sent afterward in that window.

- Selective Repeat ARQ: Optimized for noisy channels to preserve bandwidth by retransmitting only the specific frames that were lost or damaged.

  - Windows: The Send Window and Receive Window must share the exact same size, capped at a maximum of $2^{m-1}$ (half the sequence number space).

  - Behavior: The receive window allows out-of-order frames to be accepted and held in a temporary buffer until missing frames arrive to complete a sequential block.

Group C: Multiple Access Protocols

1. CSMA/CD (Collision Detection)

- Mechanism: Used primarily in wired networks, CSMA/CD expands on standard CSMA by adding a collision monitoring phase.

- Operation: A station monitors the medium while it is transmitting. If it finishes without detecting an anomaly, transmission succeeds. If a collision is detected, it aborts transmission immediately, broadcasts a jamming signal, and schedules a retry using back-off rules.

- Frame Size Restriction: For CSMA/CD to accurately catch collisions before a frame completely leaves the station, the frame transmission time ($T_{fr}$) must be at least two times the maximum propagation time ($2 \times T_p$).

2. CSMA/CA (Collision Avoidance)

- Mechanism: Developed specifically for wireless networks where collision detection is difficult due to the "hidden station problem".

- Three Avoidance Strategies: Avoids collisions using three structural layers:

  - Interframe Space (IFS): When the channel is found idle, the station defers transmission for a brief period called the IFS to allow priority traffic or clear channels.

  - Contention Window: A soft, slot-based timer where a station picks a random number of slots to wait. Crucial Rule: If the station detects the channel becomes busy during this countdown, it freezes its timer, resuming it only after the channel becomes idle again.

  - Acknowledgments (ACK): Because collisions aren't actively monitored on the line, an explicit ACK from the receiver is required to verify a successful transmission.

3. Token Passing

- Mechanism: A controlled-access method where stations organize into a logical ring structure, regardless of their physical network layout.

- Operation: Access to the shared channel is dictated by a unique control frame known as a token. A station is only authorized to send data when it receives and holds this token; when it finishes transmitting, it passes the token to its successor in the ring.

4. Polling

- Mechanism: Designed for topologies with a strict master-slave configuration, composed of one Primary Station and multiple Secondary Stations.

- Functions: The primary station completely coordinates link access via two processes:

  - Poll Function: If the primary station is ready to receive data, it explicitly asks the secondary devices one-by-one if they have anything to send.

  - Select Function: If the primary station needs to send data, it alerts a chosen secondary device to prepare its buffers to receive incoming data.

5. Reservation

- Mechanism: A controlled-access protocol where stations must explicitly allocate access parameters before placing data on the link.

- Operation: Channel time is broken into intervals, where a reservation frame directly precedes the actual data frames. If there are $N$ stations in the network, the reservation frame contains exactly $N$ mini-slots. Each station owns a specific mini-slot; if it needs to transmit data, it sets a bit in its slot. Once the reservation frame passes, the stations that flagged reservations transmit their data frames in order.