1. Digital Transmission & Line Coding

- Digital transmission = sending 0s & 1s (or digitized analog signals) over a medium.

- Line coding = converting 0s & 1s into voltage/pulse signals for the medium.

Mini tip: “Data → Signal → Medium”

2. Line Coding Characteristics

- Signal levels vs Data levels → signal = voltages, data = # of symbols

- Pulse rate vs Bit rate → (\text{Bit rate} = \text{Pulse rate} × \log_2 L)

- DC component → unwanted average voltage; bad for transformers.

- Self-synchronization → enough transitions so receiver can track bits.

Mini trick: Think: “Clock follows the pulses” ⏰

3. Math Example – Pulse Rate vs Bit Rate

- Pulse duration = 1 ms → Pulse rate = 1/0.001 = 1000 pulses/sec

- 4 levels → log₂4 = 2 → Bit rate = 1000 × 2 = 2000 bps

Quick memory: Pulse = heartbeats, Bit = info inside each heartbeat 💓

4. Line Coding Schemes

| Type | How | Example | Notes |
| --- | --- | --- | --- |
| Unipolar | Only 0 & +V | NRZ unipolar | DC + no sync |
| Polar | +V / −V | NRZ, RZ | RZ self-clock |
| Manchester | Mid-bit transition | High→Low =1 | Self-sync |
| Diff Manchester | Always mid-bit transition | Start transition = 0 |  |
| Bipolar | +, −, 0 | AMI, Pseudoternary | No DC, alternating 1s/0s |

Trick: AMI = “1s alternate voltage”, Pseudoternary = “0s alternate voltage”

5. PCM, Sampling

- PCM = Analog → Sample → Quantize → Encode → Digital

- Sampling interval (T) → time between samples

- Sampling rate (fs) → 1/T, must ≥ 2 × fmax (Nyquist)

Memory tip: “Twice the max frequency, or you’ll miss info 👀”

6. Sampling Math Examples

- Signal 0–4 kHz → fs = 2 × 4000 = 8000 Hz

- 8 bits/sample → Bit rate = 8000 × 8 = 64 kbps

- Signal 1–11 kHz → fs = 2 × 11,000 = 22,000 Hz

7. Transmission Modes

- Parallel: multiple wires → fast, expensive (computer buses)

- Serial: 1 wire → slow, cheap (USB, Ethernet)

8. Transmission Timing

| Mode | Start/Stop bits? | Speed | Example |
| --- | --- | --- | --- |
| Async | Yes | Low | Keyboard → PC |
| Sync | No | High | Ethernet |

Mini tip: Async = “byte by byte”, Sync = “continuous stream”

✅ Pro tip for your quiz today:

- Remember formulas: Bit rate = Pulse rate × log₂L

- Remember Nyquist: fs ≥ 2 × fmax

- Line codes: just remember who has DC, who self-clocks