Assignment on Line Coding Techniques

Course: EEE-301 – Data and Telecommunication
Chapter: Digital Transmission (Chapter-4)
Topic: Bit to Digital Signal Conversion (Line Coding)

Problem Statement

Determine the digital signal for the following binary data using:

- Unipolar (NRZ)

- RZ (Return to Zero)

- NRZ-L

- NRZ-I

- Manchester

- Differential Manchester

Given data:
(i) 1011001001
(ii) 11011110011

Basic Assumptions (As per PDF)

- Positive voltage = +V

- Negative voltage = –V

- Zero voltage = 0

- Initial signal level (for NRZ-I & Differential Manchester) = Low (–V)

1. Unipolar NRZ Encoding

Definition (from PDF):
In Unipolar encoding, only one voltage level is used.
Binary 1 is represented by +V and binary 0 by 0 voltage.

(i) 1011001001

| Bit | 1 | 0 | 1 | 1 | 0 | 0 | 1 | 0 | 0 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Signal | +V | 0 | +V | +V | 0 | 0 | +V | 0 | 0 | +V |

(ii) 11011110011

| Bit | 1 | 1 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 1 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Signal | +V | +V | 0 | +V | +V | +V | +V | 0 | 0 | +V | +V |

2. RZ (Return to Zero) Encoding

Definition:
Each bit period is divided into two halves.

- 1 → +V then 0

- 0 → –V then 0

(i) 1011001001

| Bit | 1 | 0 | 1 | 1 | 0 | 0 | 1 | 0 | 0 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Signal | +V→0 | –V→0 | +V→0 | +V→0 | –V→0 | –V→0 | +V→0 | –V→0 | –V→0 | +V→0 |

(ii) 11011110011

Signal Pattern

+V→0, +V→0, –V→0, +V→0, +V→0, +V→0, +V→0, –V→0, –V→0, +V→0, +V→0

3. NRZ-L (Non-Return-to-Zero Level)

Definition:

- Binary 1 → –V

- Binary 0 → +V

(i) 1011001001

| Bit | 1 | 0 | 1 | 1 | 0 | 0 | 1 | 0 | 0 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Signal | –V | +V | –V | –V | +V | +V | –V | +V | +V | –V |

(ii) 11011110011

Signal

–V, –V, +V, –V, –V, –V, –V, +V, +V, –V, –V

4. NRZ-I (Non-Return-to-Zero Invert)

Definition:

- 1 → Invert signal

- 0 → No change

(i) 1011001001

(Initial level = –V)

| Bit | 1 | 0 | 1 | 1 | 0 | 0 | 1 | 0 | 0 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Level | +V | +V | –V | +V | +V | +V | –V | –V | –V | +V |

(ii) 11011110011

Level

+V, –V, –V, +V, –V, +V, –V, –V, –V, +V, –V

5. Manchester Encoding

Definition:
There is always a transition in the middle of the bit interval.

- 1 → Low to High

- 0 → High to Low

(i) 1011001001

| Bit | 1 | 0 | 1 | 1 | 0 | 0 | 1 | 0 | 0 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Signal | LH | HL | LH | LH | HL | HL | LH | HL | HL | LH |

(ii) 11011110011

Signal

LH, LH, HL, LH, LH, LH, LH, HL, HL, LH, LH

6. Differential Manchester Encoding

Definition:

- Always a transition at the middle

- 0 → Transition at the beginning

- 1 → No transition at the beginning

(i) 1011001001

(Initial level = High)

| Bit | 1 | 0 | 1 | 1 | 0 | 0 | 1 | 0 | 0 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rule | NT | T | NT | NT | T | T | NT | T | T | NT |

(ii) 11011110011

Rule

NT, NT, T, NT, NT, NT, NT, T, T, NT, NT

Conclusion

Different line coding techniques represent binary data using different voltage patterns.

- Unipolar is simple but suffers from DC component

- RZ improves synchronization

- NRZ-L and NRZ-I are bandwidth efficient

- Manchester and Differential Manchester provide self-synchronization and are widely used in data communication systems