Chapter 4 – Digital Transmission (Forouzan).

1. Digital Transmission (5 Marks)

Definition:
Digital transmission is the transfer of digital data (bitstream or digitized analog signal) over a communication channel.

Communication Channels:

- Copper wires

- Optical fibers

- Wireless links

- Storage media

- Computer buses

Signal Representation:
Data is represented as electromagnetic signals such as:

- Electrical voltage

- Radio waves

- Microwave

- Infrared signals

Advantages:

- More reliable than analog transmission

- Resistant to noise

- Supports error detection and correction

Signal Level vs. Data Level (Comparison Table – Exam Standard)

| Basis of Comparison | Signal Level | Data Level |
| --- | --- | --- |
| Definition | Number of distinct voltage (or amplitude) values allowed in a signal. | Number of different symbols used to represent data. |
| Concerned With | Physical representation of the signal. | Logical representation of data (bits or symbols). |
| Represents | Voltage, current, or signal amplitude. | Binary digits (0,1) or multilevel data symbols. |
| Relation to Transmission | Determines how the signal appears on the transmission medium. | Determines how many bits are carried per symbol. |
| Example (Binary) | 2 signal levels → +V and −V | 2 data levels → 0 and 1 |
| Example (Multilevel) | 3 signal levels (−V, 0, +V) | May still represent 2 data levels |
| Formula Relation | Used in Bit Rate formula through L (data levels). | L = number of data levels in BitRate = PulseRate × log₂L |
| Exam Point | Physical characteristic of line coding. | Logical meaning of transmitted information. |

Pulse Rate vs. Bit Rate (Comparison Table – 5 Marks)

| Basis of Comparison | Pulse Rate | Bit Rate |
| --- | --- | --- |
| Definition | Number of pulses transmitted per second. | Number of bits transmitted per second. |
| Unit | Pulses per second (baud) | Bits per second (bps) |
| Represents | Speed of signal changes on the transmission medium. | Speed of actual data transmission. |
| Depends On | Pulse duration (minimum time to send one symbol). | Pulse rate and number of data levels (L). |
| Formula | Pulse Rate = 1 / Pulse Duration | Bit Rate = Pulse Rate × log₂L |
| When L = 2 | Pulse rate = Bit rate | Because log₂2 = 1 |
| When L > 2 | Pulse rate < Bit rate | Because one pulse carries multiple bits |

Example (Important for Exam)

If pulse duration = 1 ms and L = 4

Pulse Rate = 1 / 0.001 = 1000 pulses/sec
Bit Rate = 1000 × log₂4 = 1000 × 2 = 2000 bps

4. Unipolar Encoding (5 Marks)

Definition:
Unipolar encoding uses only one voltage level to represent data.

Representation:

- Binary 1 → High voltage

- Binary 0 → No voltage

Advantages:

- Simple implementation

Disadvantages:

- Large DC component

- Poor synchronization

- Not suitable for long-distance communication

5. Polar Encoding (5 Marks)

Definition:
Polar encoding uses two voltage levels: positive and negative.

Types:

- NRZ

- RZ

- Manchester

- Differential Manchester

Advantages:

- Eliminates DC component

- Better synchronization than unipolar

6. NRZ (Non-Return to Zero) (5 Marks)

Definition:
NRZ uses two voltage levels to represent binary values.

Representation:

- Positive voltage → 0

- Negative voltage → 1

Advantages:

- Simple

- Efficient bandwidth usage

Disadvantages:

- Synchronization problem with long identical bits

7. RZ (Return to Zero) (5 Marks)

Definition:
Signal returns to zero between each bit.

Representation:

- Bit 1 → Positive → Zero

- Bit 0 → Negative → Zero

Advantages:

- Ensures synchronization

Disadvantages:

- Requires 3 amplitude levels

- Needs more bandwidth

8. Manchester Encoding (5 Marks)

Definition:
Self-synchronizing encoding with a transition in the middle of each bit.

Representation:

- 1 → High to Low

- 0 → Low to High

Advantages:

- No DC component

- Easy clock recovery

Disadvantages:

- Requires double bandwidth compared to NRZ

Application: Used in Ethernet networks.

9. Differential Manchester (5 Marks)

Definition:
Always has a transition in the middle of the bit interval.

Representation:

- Transition at beginning → 0

- No transition at beginning → 1

Advantages:

- Self-synchronizing

- Immune to polarity inversion

Application: Used in Token Ring networks.

10. Bipolar Encoding (5 Marks)

Definition:
Uses three voltage levels: positive, negative, and zero.

Types:

- AMI (Alternate Mark Inversion)

  - 0 → Zero voltage

  - 1 → Alternating + and −

- Pseudoternary

  - 1 → Zero voltage

  - 0 → Alternating + and −

Advantages:

- Reduces DC component

Disadvantages:

- Synchronization problem with long sequence of 0s

11. Pulse Amplitude Modulation (PAM) (5 Marks)

Definition:
PAM is an analog-to-digital conversion method where sampled signal amplitudes are represented by pulses.

Process:

- Sampling

- Quantization

- Encoding (in PCM)

Note:
PAM is the first step in Pulse Code Modulation (PCM).

Parallel vs Serial Transmission (5 Marks – Exam Standard)

| Basis of Comparison | Parallel Transmission | Serial Transmission |
| --- | --- | --- |
| Definition | Multiple bits are transmitted simultaneously over multiple channels. | Bits are transmitted one after another over a single channel. |
| Number of Wires | Requires multiple wires (equal to number of bits). | Requires only one communication line. |
| Speed | High speed (short distance). | Slower than parallel (but efficient for long distance). |
| Cost | Expensive due to multiple wires. | Cheaper and simpler. |
| Distance | Suitable for short distances. | Suitable for long distances. |
| Example | Used inside computer (CPU to memory, system bus). | Used in network communication, USB, internet data transfer. |
| Interference | More prone to crosstalk and signal skew. | Less interference problem. |

Short Conclusion (Write in Exam)

Parallel transmission sends multiple bits at the same time using multiple wires and is used for short distances. Serial transmission sends bits sequentially over a single line and is suitable for long-distance communication.

Asynchronous vs Synchronous Transmission (5 Marks – Exam Standard)

| Basis of Comparison | Asynchronous Transmission | Synchronous Transmission |
| --- | --- | --- |
| Definition | Data is transmitted one byte at a time with start and stop bits. | Data is transmitted as a continuous stream of bits without start/stop bits. |
| Synchronization | Synchronization at byte level. | Synchronization at frame or block level. |
| Start/Stop Bits | Each byte has 1 start bit (0) and 1 or more stop bits (1). | No start/stop bits used. |
| Gap Between Data | May have gaps between bytes. | No gaps between bits. |
| Efficiency | Less efficient (extra bits added). | More efficient (no extra start/stop bits). |
| Complexity | Simple implementation. | Complex, requires precise clock synchronization. |
| Example | Keyboard input, serial port communication. | High-speed communication, LAN, WAN. |

Short Conclusion (Write in Exam)

Asynchronous transmission sends data byte by byte with start and stop bits, making it simple but less efficient. Synchronous transmission sends a continuous stream of bits without start/stop bits, making it faster and more efficient but requiring proper clock synchronization.