Lecture 01 – Data Communication PDF

📡 1. Define Data Communication

Data communication is the exchange of data between two devices via a transmission medium like wire or cable.

⚙️ 2. Characteristics of Data Communication

- Delivery: Data must reach the correct destination.

- Accuracy: Data must be delivered without errors.

- Timeliness: Data must arrive on time (especially for audio/video).

- Jitter: Jitter refers to the variation in the packet arrival time. It is the uneven delay in the delivery of audio or video packets.

🧩 3. Components of Data Communication

- Message – The data (text, audio, video, etc.)

- Sender – The sender is the device that sends the data message

- Receiver – The receiver is the device that receives the message.

- Transmission Medium – The transmission medium is the physical path by which a message travels from sender to receiver. (e.g., cable, radio)

- Protocol – A protocol is a set of rules that govern data communications.

🔁 4. Classification by Data Flow

- Simplex – In simplex mode, the communication is unidirectional, as on a one-way street. Only one of the two devices on a link can transmit; the other can only receive  (e.g., keyboard to monitor)

- Half-Duplex – In half-duplex mode, each station can both transmit and receive, but not at the same time. When one device is sending, the other can only receive, and vice versa. (e.g., walkie-talkie)

- Full-Duplex – In full-duplex mode (also called duplex), both stations can transmit and receive simultaneously . The full-duplex mode is like a two-way street with traffic flowing in both directions at the same time. (e.g. teliphone)

🌐 5. Define Computer Network

A network is a set of devices (often referred to as nodes) connected by communication links.

📊 6. Criteria of a Computer Network

- Performance – Can be measured in many ways, including transit time and response time.

- Transit time is the amount of time required for a message to travel from one device to another device.

- Response time is the elapsed time between an inquiry and a response.

- Reliability – In addition to accuracy of delivery, network reliability is measured by the frequency of failure, the time it takes a link to recover from a failure and the network's robustness in a  calamity. [Failure rate, recovery time]

- Security – Protection from unauthorized access and damage

🔗 7. Types of Connections

- Point-to-Point – Dedicated link between two devices

- Multipoint – Shared link among multiple devices

🕸️ 8. Topologies with Figures, Advantages & Disadvantages

- Mesh – In a mesh topology, every device has a dedicated point-to-point link to every other device. The term dedicated means that the link carries traffic only between the two devices it connects. [Every device connects to every other]

  - ✅ Robust, private, fault isolation

  - ❌ Expensive, complex installation

- Star – In a star topology, each device has a dedicated point-to-point link only to a central controller, usually called a hub (or switch) [Devices connect to a central hub]

  - ✅ Easy to manage, fault isolation

  - ❌ Hub failure affects entire network

- Bus – One long cable acts as a backbone to link all the devices in a network. Nodes are connected to the bus cable by drop lines and taps. [Single backbone cable]

  - ✅ Simple, less cabling

  - ❌ Fault affects all devices, hard to reconfigure

- Ring – In a ring topology, each device has a dedicated point-to-point connection with only the two devices on either side of it. A signal is passed along the ring in one direction, from device to device, until it reaches its destination. Each device in the ring incorporates a repeater. When a device receives a signal intended for another device, its repeater regenerates the bits and passes them along. [Devices form a closed loop]

  - ✅ Easy to add/remove, fault detection

  - ❌ One break disables network

- Hybrid – Combination of topologies.

🏙️ 9. Demonstrate LAN, MAN, WAN

- LAN – A local area network (LAN) is usually privately owned and links the devices in a single office, building, or campus.

- MAN – A metropolitan area network (MAN) is a network with a size between a LAN and a WAN. It normally covers the area inside a town or a city.

- WAN – A wide area network (WAN) provides long-distance transmission of data, image, audio and video information over large geographic areas that may comprise a country, a continent, or even the whole world.

🔍 10. Differences Among LAN, MAN, WAN

| Feature | LAN | MAN | WAN |
| --- | --- | --- | --- |
| Range | 1–10 km | ~100 km | Beyond 100 km |
| Media Used | Guided | Guided & Unguided | Unguided |
| Speed | High (100 kbps–100 Mbps) | Moderate | Lower than LAN |
| Cost | Cheap | Costly | Expensive |
| Equipment | NIC, switch, hub | Modem, router | Microwave, radio, laser |
| Protocols | ARCNET, Token Ring | Frame Relay, ATM | ATM, FDDI, SMDS |

📜 11. Protocol and Key Elements

- Protocol – A protocol is a set of rules that govern data communications. A protocol defines what is communicated, how it is communicated, and when it is communicated.

- Key Elements:

  - Syntax – The term syntax refers to the structure or format of the data, meaning the order in which they are presented.

  - Semantics – The word semantics refers to the meaning of each section of bits. How is a particular pattern to be interpreted and what action is to be taken based on that interpretation.

  - Timing – The term timing refers to two characteristics: when data should be sent and how fast they can be sent.