1-Mark Questions (Definitions & Key Terms)

These questions typically require a one-sentence answer or a simple identification.

- Define Digital Transmission: The transfer of a digital bitstream or digitized analog signal over a channel.

- Define Line Coding: The process of converting digital data into digital signals.

- What is a Carrier Signal? A high-frequency signal that carries information through space as an electromagnetic wave.

- Define Multiplexing: A method where multiple signals are combined into one signal over a shared medium.

- What is Attenuation? The loss of signal energy as it travels through a medium.

- Define Single-Bit Error: When only one bit in a data unit has changed from 0 to 1 or vice-versa.

2-Mark Questions (Short Explanations & Basic Concepts)

- Signal Level vs. Data Level: Signal level is the number of different values allowed in a signal, while data level is the number of symbols representing data.

- Bit Rate vs. Baud Rate: Bit rate is the number of bits per second; baud rate is the number of signal units per second required to represent those bits.

- Transmission Impairments: List and briefly define attenuation (energy loss), distortion (change in shape), and noise (signal corruption).

- Redundancy in Error Detection: The concept of adding extra bits to data to help the destination detect errors.

- Guided vs. Unguided Media: Guided media use physical cables (copper/fiber) to guide signals, while unguided media provide wireless transmission through space.

4-Mark Questions (Comparisons & Mid-level Calculations)

- Parallel vs. Serial Transmission: Describe how parallel sends multiple bits simultaneously over multiple lines (high speed, high cost), whereas serial sends them one after another over one line.

- Asynchronous vs. Synchronous Transmission: Explain that asynchronous uses start/stop bits for each byte, while synchronous sends bits in a continuous stream without gaps.

- Mathematical Problems (Pulse/Bit Rate): Be prepared to calculate Bit Rate using the formula: $BitRate = PulseRate \times \log_2 L$ (where $L$ is the number of data levels).

- FDM vs. TDM: Contrast Frequency-Division Multiplexing (dividing frequency bands, uses guard bands) with Time-Division Multiplexing (dividing time slots, uses framing bits).

- Types of Radio Wave Propagation: Describe Ground propagation (between earth and ionosphere), Sky propagation (reflected off ionosphere), and Line-of-sight propagation.

5-Mark Questions (Detailed Descriptions, Diagrams & Complex Math)

- Line Coding Schemes: Describe and provide examples for Unipolar, Polar (NRZ, RZ, Manchester, Differential Manchester), and Bipolar (AMI and Pseudoternary).

- Digital-to-Analog Modulation: Explain the mechanisms of Amplitude Shift Keying (ASK), Frequency Shift Keying (FSK), and Phase Shift Keying (PSK) with their advantages and disadvantages.

- Circuit Switching vs. Packet Switching: Provide a detailed comparison focusing on connection orientation (Circuit is connection-oriented; Packet is connectionless), flexibility, and layer implementation.

- Fiber Optic Cable: Discuss the characteristics (single mode vs. multi-mode), advantages (high bandwidth, noise resistance), and disadvantages (cost, fragility) of optical fibers.

- Mathematical Problems (Nyquist Sampling Rate): Calculate required sampling rates for digitization (Human voice example: $4000\text{ Hz} \times 2 = 8000\text{ samples/s}$) and resulting bit rates.

- Error Detection Techniques (CRC/Checksum): Be ready to perform a Cyclic Redundancy Check (CRC) calculation using binary division or a Checksum addition using one's complement.

Final Study Tips

- Draw Diagrams: For 4 or 5-mark questions regarding PCM, Multiplexing, or Switching, always include the relevant diagrams provided in the lectures.

- Focus on "Questions for Exams" Slid: Chapter 4 has a specific list of 8 exam questions that are highly likely to appear.

- Check Units: In math problems, ensure you correctly convert between bits, bytes, bps, and Kbps.

🔑 High-Probability Exam Topics (Based on Your PDFs)

⚡ Must-Know Math Problems (Practice These!)

- Bit Rate vs. Baud Rate (Ch5):

  - Formula: Bit rate = Baud rate × log₂L where L = signal levels

  - Example: "If baud rate = 1000 and each signal unit carries 4 bits, find bit rate" → 4000 bps

  - Reverse: "Bit rate = 3000 bps, 6 bits/signal → Baud rate = 500"

- Nyquist Sampling Theorem (Ch4):

  - Formula: Sampling rate ≥ 2 × highest frequency

  - Example: "Human voice (0-4000 Hz) with 8 bits/sample → Bit rate = 8000 × 8 = 64 Kbps"

- FDM Bandwidth Calculation (Ch6):

  - Total bandwidth = (N × channel BW) + [(N-1) × guard band]

  - Example: "5 channels × 100 KHz + 4 guard bands × 10 KHz = 540 KHz"

- TDM Frame Calculations (Ch6):

  - Frame size = N × bits per channel

  - Frame duration = 1 / (data rate per channel)

  - Bit rate of link = N × bit rate per channel

📐 Essential Diagrams to Practice Drawing

| Chapter | Diagram | Why Important |
| --- | --- | --- |
| Ch4 | Manchester & Differential Manchester encoding waveforms | Very common 4-5 mark question |
| Ch5 | ASK, FSK, PSK signal waveforms | Core modulation schemes |
| Ch5 | 8-QAM constellation diagram | Shows combined amplitude/phase modulation |
| Ch6 | FDM multiplexer/demultiplexer block diagram | Shows carrier modulation + guard bands |
| Ch6 | TDM frame structure with time slots | Shows interleaving concept |
| Ch8 | Fiber optic cable cross-section (core/cladding) | Guided media fundamentals |

📝 Theory Topics by Mark Weight

1-2 Mark Questions (Define/Short Answer):

- DC component in line coding (Ch4)

- Guard bands in FDM vs. framing bits in TDM (Ch6)

- Circuit vs. packet switching differences (Ch7)

- Attenuation vs. distortion vs. noise (Ch8)

- OOK (On-Off Keying) as ASK variant (Ch5)

4-5 Mark Questions (Explain with Diagrams):

- Compare ASK, FSK, PSK (advantages/disadvantages) – include noise susceptibility

- PCM process: sampling → quantization → encoding (Ch4)

- FDM working principle with multiplexer/demultiplexer steps (Ch6)

- Twisted pair vs. coaxial vs. fiber optic (table comparison of characteristics) (Ch8)

- Circuit switching 3 phases: setup → data transfer → teardown (Ch7)

Here's your **chapter-wise exam preparation guide** based strictly on your PDFs. I'll focus on high-yield topics, math problems *with solutions from your lectures*, and diagrams you must practice. *(Note: No one can guarantee "100% common questions"—but mastering these gives you 45+/50 marks reliably.)*

---------------------------

📚 Chapter 4: Digital Transmission

🔢 Must-Practice Math (From PDF Examples)

1. **Pulse Rate vs Bit Rate**

*Q: A signal has 4 data levels with pulse duration 1 ms. Find pulse & bit rates.*

→ Pulse rate = 1 / 0.001s = **1000 pulses/s**

→ Bit rate = 1000 × log₂4 = **2000 bps**

2. **Nyquist Sampling**

*Q: Human voice (0–4000 Hz), 8 bits/sample → bit rate?*

→ Sampling rate = 2 × 4000 = **8000 samples/s**

→ Bit rate = 8000 × 8 = **64 Kbps** *(This exact example is in your PDF!)*

### ✏️ Diagrams to Draw (4–5 mark questions)

- Manchester encoding waveform (self-synchronizing, 2 levels)

- Differential Manchester (transition in middle = always; beginning transition = 0)

- PCM process: **Analog → Sampling → Quantization → Encoding → Digital**

### 📝 Theory by Marks

| Marks | Topic |

|-------|-------|

| 1–2 | DC component definition (avg. amplitude > 0 = bad for transformers) |

| 2 | Why Manchester solves synchronization problem |

| 4–5 | Compare NRZ vs RZ vs Manchester (include bandwidth & synchronization) |

---

## 📚 Chapter 5: Analog Transmission

### 🔢 Must-Practice Math (From PDF Examples)

1. **Bit Rate vs Baud Rate**

*Q: 4 bits/signal unit, 1000 signal units/sec → baud & bit rates?*

→ Baud rate = **1000 baud/s**

→ Bit rate = 1000 × 4 = **4000 bps** *(Directly from your PDF)*

*Q: Bit rate = 3000 bps, 6 bits/signal → baud rate?*

→ Baud rate = 3000 / 6 = **500 baud/s**

### ✏️ Diagrams to Draw

- ASK waveform (amplitude changes for 0/1)

- FSK waveform (frequency changes)

- PSK waveform (phase shift at bit boundaries)

- **8-QAM constellation diagram** (4 amplitude levels × 2 phases = 8 points)

### 📝 Theory by Marks

| Marks | Topic |

|-------|-------|

| 1–2 | OOK definition (On-Off Keying = ASK variant with 0V for '0') |

| 2 | Why PSK > ASK in noise resistance |

| 4–5 | Compare ASK/FSK/PSK: noise susceptibility, bandwidth, applications |

---

## 📚 Chapter 6: Multiplexing

### 🔢 Must-Practice Math (From PDF Examples)

1. **FDM with Guard Bands**

*Q: 5 channels × 100 KHz each + 10 KHz guard bands → total bandwidth?*

→ Total = (5 × 100) + (4 × 10) = **540 KHz** *(Exact PDF example)*

2. **TDM Frame Calculation**

*Q: 4 channels, 100 bytes/s each, 1 byte/channel per frame → frame size, duration, bit rate?*

→ Frame size = 4 bytes = **32 bits**

→ Frame duration = 1 / 100 = **0.01 s**

→ Bit rate = 4 × 100 × 8 = **3200 bps**

### ✏️ Diagrams to Draw

- FDM multiplexer block diagram (modulators → combiner → transmission link)

- TDM frame structure (time slots for each channel + framing bits)

- WDM prism combining/separating wavelengths

### 📝 Theory by Marks

| Marks | Topic |

|-------|-------|

| 2 | Guard bands (FDM) vs framing bits (TDM) purpose |

| 4–5 | FDM vs TDM comparison table (analog/digital, bandwidth efficiency, complexity) |

---

## 📚 Chapter 7: Switching

### 📝 Theory by Marks (No math in this chapter)

| Marks | Topic |

|-------|-------|

| 1–2 | Circuit switching 3 phases: **setup → data transfer → teardown** |

| 2 | Message switching = store-and-forward |

| 4–5 | Circuit vs Packet switching comparison:

- Connection-oriented vs connectionless

- Dedicated path vs dynamic routing

- In-order delivery vs out-of-order packets

- Physical layer vs network layer implementation |

---

## 📚 Chapter 8: Transmission Media

### 📝 Theory by Marks

| Marks | Topic |

|-------|-------|

| 1–2 | Attenuation vs distortion vs noise definitions |

| 2 | 4 noise types: thermal, induced, crosstalk, impulse |

| 4–5 | **Fiber optic advantages** (must list 5+):

1. Higher bandwidth

2. Less attenuation (50 km vs 5 km for copper)

3. Noise immunity (light vs electricity)

4. Security (no electromagnetic tapping)

5. Lighter weight (9 lbs/1000 ft vs 80 lbs for copper) |

### ✏️ Diagrams to Draw

- Fiber optic cross-section (core/cladding/jacket)

- Twisted pair cable showing different twist lengths to reduce crosstalk

- Propagation types table:

| Propagation | Frequency Range | Application |

|-------------|-----------------|-------------|

| Ground | VLF/LF/MF | AM/FM radio |

| Sky | 2–30 MHz | Shortwave radio |

| Line-of-sight | HF/VHF/UHF | Satellite, cellular |