Booth's Multiplication Algorithm

Booth's Multiplication Algorithm is an efficient algorithm used for multiplying signed binary numbers. It minimizes the number of arithmetic operations by handling runs of 1s in the multiplier efficiently. Below is a detailed explanation with steps and illustrations.

Key Concepts of Booth’s Algorithm

- It works on 2’s complement binary numbers, allowing signed multiplication.

- A multiplier bit and a previous bit (Q[-1]) are considered to decide the operation.

- The algorithm involves adding/subtracting the multiplicand based on specific conditions.

Steps in Booth’s Algorithm

Let:

- A = Accumulator (holds partial results, initialized to 0)

- Q = Multiplier (initialized with the multiplier)

- Q[-1] = Extra bit for the multiplier, initialized to 0

- M = Multiplicand

- n = Number of bits in the operands

- Initialize:

  - Set A = 0, Q[-1] = 0.

  - Load the multiplicand in M and the multiplier in Q.

- Check conditions: For each step (repeated n times):

  - If Q[0] = 0 and Q[-1] = 1: Add the multiplicand to A.

  - If Q[0] = 1 and Q[-1] = 0: Subtract the multiplicand from A.

  - Otherwise, no operation on A.

- Perform arithmetic shift right (ASR):

  - Shift the combined register (A, Q, Q[-1]) right by 1 bit.

  - Sign-extend A during the shift.

- Repeat for n cycles.

- Final result:

  - Combine A and Q to get the final product.

Example

Multiply 3 (00110011) and -4 (11001100) in 4 bits using Booth’s Algorithm.

Initialization

M = 0011 (3 in binary)

-M = 1101 (2’s complement of M)

Q = 1100 (-4 in binary)

Q[-1] = 0

A = 0000

Steps

| Step | A | Q | Q[-1] | Operation | Explanation |
| --- | --- | --- | --- | --- | --- |
| 0 | 0000 | 1100 | 0 | Initial state |  |
| 1 | 1101 | 1100 | 0 | A = A - M | Subtract M (Q[0] = 1, Q[-1] = 0) |
|  | 1110 | 1110 | 0 | ASR | Arithmetic Shift Right |
| 2 | 1110 | 1110 | 1 | No operation | (Q[0] = 1, Q[-1] = 1) |
|  | 1111 | 0111 | 1 | ASR |  |
| 3 | 0011 | 0111 | 1 | A = A + M | Add M (Q[0] = 0, Q[-1] = 1) |
|  | 0001 | 1011 | 1 | ASR |  |
| 4 | 1110 | 1011 | 1 | A = A - M | Subtract M (Q[0] = 1, Q[-1] = 0) |
|  | 1111 | 1101 | 1 | ASR |  |

Final Result

Combine AA and QQ:
Result = A∣∣Q=1111A || Q = 1111 in 8 bits = -12 in decimal.